pyhula

A Python pack used by hula.

Python version:3.12.10

Installing

Input the following code in powershell(cmd.exe) to install pyhula.

pip install pyhula

pip install pyhula-1.1.7-cp36-cp36m-win_amd64.whl

Missing libraries

If you get error message "missing some libraries", please install them by the following code.

pip install opencv-python install_requires=[

"psutil",

"numpy",

"opencv-python", "pymavlink"

],

Checking version

Input "pip list" in powershell(cmd.exe) to get pyhula's verison

Using the following code.

import pyhula

ver = pyhula.get_version() print(ver)

Usage

Use the following codes to create a userApi instance. Its interfaces can be used to control fylo plane.

import pyhula

api = pyhula.UserApi() if not api.connect():

print("connect error") else:

print('connection to	station by wifi')

api.single_fly_takeoff() #takeoff

api.single_fly_touchdown()	# Landing

Interface

Connected drone

connect(server_ip) '''

description:

connect drone,

parameter :

optional:server_ip: If  the drone IPv4 address is not specified,

the return value is automatically obtained:

True: True  False: False

'''

Example code: api.connect('192.168.1.118')

Example code: api.connect()

Takeoff

single_fly_takeoff(led) '''

description:

Control drone takeoff  in real time

parameter :

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_takeoff() api.single_fly_takeoff({'r':16,'g':15,'b':100,'mode':1})

Landing

single_fly_touchdown(led) '''

description:

Control drone Landing in real time

parameter :

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code :api.single_fly_touchdown() api.single_fly_touchdown({'r':16,'g':15,'b':100,'mode':1})

Hover

single_fly_hover_flight(time,led) '''

description:

hover_flight

parameter:

time: Hover time (seconds)

'''

Example code : api.single_fly_hover_flight(10) api.single_fly_hover_flight(10,{'r':16,'g':15,'b':100,'mode':1})

Fly_forward

single_fly_forward(distance,speed,led)

'''

description:

Control drone fly_forward in real time

parameter:

distance: Flight distance (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_forward(100)

api.single_fly_forward(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Fly_backward

single_fly_back(distance,speed,led)

'''

description:

Control drone  fly_backward  in real time

parameter:

distance: Flight distance (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s                                                                        led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_back(100)

api.single_fly_back(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Fly to left

single_fly_left(distance,speed,led)

'''

description:

Control drone  fly to left  in real time

parameter:

distance:  Flight distance (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_left(100)

api.single_fly_left(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Fly to right

single_fly_right(distance, speed,led)

'''

description:

Control drone  fly to right  in real time

parameter:

distance: Flight distance (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s                                                                      led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_right(100)

api.single_fly_right(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Fly_up

single_fly_up(height, speed,led)

'''

description:

Control drone  fly_up  in real time

parameter:

height:Flight height (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_up(100)

api.single_fly_up(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Fly_down

single_fly_down(height,speed,led)

'''

description:

Control drone fly_down in real time

parameter:

height: Flight height (cm)

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_down(100)

api.single_fly_down(100, 100,{'r':16,'g':15,'b':100,'mode':1})

Turn left

single_fly_turnleft(angle,led) '''

description:

Control drone turn left in real time

parameter:

angle: Rotation Angle (degree)

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_turnleft(90)

api.single_fly_turnleft(90,{'r':16,'g':15,'b':100,'mode':1})

Turn right

single_fly_turnright(angle,led) '''

description:

Control drone turn right in real time

parameter:

angle: Rotation Angle (degree)

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_turnright(90) api.single_fly_turnright(90,{'r':16,'g':15,'b':100,'mode':1})

Bounce

single_fly_bounce(frequency, height,led) '''

description:

Control drone  bounce  in real time

parameter:

frequency: Bounce times

height: Bounce distance (cm)

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code: api.single_fly_bounce(3, 50)

api.single_fly_bounce(3, 50,{'r':16,'g':15,'b':100,'mode':1})

Straight_flight

single_fly_straight_flight( x, y, z, speed,led)

'''

description:

straight_flight to co-ordinate (x,y,z)

parameter:

x: co-ordinate x（cm） y: co-ordinate y（cm） z: co-ordinate z（cm）

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code: api.single_fly_straight_flight(100, 100, 100)

api.single_fly_straight_flight(100, 100, 100, 50,{'r':16,'g':15,'b':100,'mode':1})

Flight around

single_fly_radius_around(radius,led) '''

description:

flight around A point ahead of the plane

parameter:

radius：radius ( cm， positive: anticlockwise ，negative:clockwise )

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_radius_around(100) api.single_fly_radius_around(100,{'r':16,'g':15,'b':100,'mode':1})

Autogyration

single_fly_autogyration360(num,led) '''

description:

Clockwise, counterclockwise rotation a certain number of turns

parameter:

num:( positive: anticlockwise ，negative:clockwise)

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_autogyration360(2) api.single_fly_autogyration360(2,{'r':16,'g':15,'b':100,'mode':1})

Somersault

single_fly_somersault(direction) '''

description:

The drone forward, backward, left or right somersault

parameter:

DIRECTION_FORWARD=0, /* forward. | */ DIRECTION_BACK=1, /* back. | */ DIRECTION_LEFT=2, /* left. | */ DIRECTION_RIGHT=3, /* right. | */

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code : api.single_fly_somersault(0) api.single_fly_somersault(0,{'r':16,'g':15,'b':100,'mode':1})

Curvilinear Flight

single_fly_curvilinearFlight(x,y,z,speed,led)

'''

description:

curvilinear Flight to (x,y,z)

parameter:

x：x co-ordinate（cm）（Body left and right, right is positive）

y：y co-ordinate（cm）（Front and back of the body, front is positive）

z：z co-ordinate（cm）（Body up and down, up is positive）

direction: True：anticlockwise

False： clockwise

Default :True

speed: If left blank, it will default to 100 speed（0-100cm）/s

led: The default value is 0，

format:{'r':0,'g':0,'b':0,'mode':1}	r,g,b: color gamut, mode: 1/ light on，2/light off，4/RGB cycle light，16/ colorful light,32/ blink light,64/ breathing light

'''

Example code: api.single_fly_curvilinearFlight(100, 100, 0 , True, 50)

api.single_fly_curvilinearFlight(100, 100, 0, False, 50,

{'r':16,'g':15,'b':100,'mode':1})

Turn on obstacle avoidance

single_fly_barrier_aircraft(mode) '''

description:

Turn on obstacle avoidance

parameter:

mode: True: Turn on  False: Turn off

'''

Example code: api.single_fly_barrier_aircraft(True)

Line_walking

single_fly_Line_walking(fun_id, dist, way_color) '''

description:

line walking

parameter:

fun_id	= 0	//0: Forward patrol，Ignore a junction

dist			// distance， cm

way_color	// Line color gamut

，0- blackest 255- White

return:

return result = 1; // The result of the instruction execution：0- failed，1- succeed ,

2- junction

'''

Example code: api.single_fly_Line_walking(0, 100, 0)

Identification label

single_fly_AiIdentifies(mode) '''

description:

identification label

parameter:

mode:

0-9： Identify numeric labels 0-9，

10： Identify left arrow，

11： Identify right arrow，

12：Identify the up arrow，

13：Identify down arrow，

20：end task，

65-90：capital letterA-Z；

The recognition process continues after the recognition is triggered 300ms，If the identification is successful, it ends immediately

return:

x; The x-coordinate of the label card relative to the drone

y; The y-coordinate of the label card relative to the drone

z; The z-coordinate of the label card relative to the drone

angle; The Angle between the label card and the drone

result；//False： failed， True： succeed

'''

Example code: api.single_fly_AiIdentifies(1)

Optical flow align QR code

single_fly_Optical_flow_alignment(qr_id, qr_size, angle = 0)

'''

description:

Optical flow align QR code

parameter:

qr_id; To QR code id[0-9]，

qr_size: The physical size of the QR code, range [6, 30], default value 20, unit :cm

angle: Align the QR code and rotate it by n degrees

return:

result；// False： failed， True： succeed

'''

Example code: api.single_fly_Optical_flow_alignment(1, 20, 0)

Optical flow recognition QR code

single_fly_Optical_flow_recognition(qr_id, qr_size)

'''

description:

Optical flow recognition QR code

parameter:

qr_id; To QR code id[0-9]，

qr_size: The physical size of the QR code, range [6, 30], default value 20, unit :cm

return:

{

result；// False： failed， True： succeed

x;// Distance between the drone and the QR code

y;// Distance between the drone and the QR code

z;// Distance between the drone and the QR code

yaw;// Angle between the drone and the QR code

qr_id;// Identified the QR code id

}

'''

Example code: api.single_fly_recognition_Qrcode(0, 1)

Front camera Align QR code

single_fly_Proactive_alignment(qr_id) '''

description:

Front camera align QR code

parameter:

qr_id; To QR code id[0-9]，

return:

result；// False： failed， True： succeed

'''

Example code: api.single_fly_Proactive_alignment(1)

Front camera recognition QR code

single_fly_Anticipatory_recognition(qr_id) '''

description:

Front camera recognition QR code

parameter:

qr_id; To QR code id[0-9]，

return:

{

result；// False： failed， True： succeed

x;// Distance between the drone and the QR code

y;// Distance between the drone and the QR code

z;// Distance between the drone and the QR code

yaw;// Angle between the drone and the QR code

qr_id;// Identified the QR code id

}

'''

Example code: api.single_fly_Anticipatory_recognition(1)

Track_Qrcode

single_fly_track_Qrcode(qr_id, time) '''

description:

Track QR code [0-9] [time] seconds

parameter:

qr_id: QR code id

time: Tracking time

return:

result：0: failed，1: succeed

'''

Example code：api.single_fly_track_Qrcode(1, 10)

single_fly_getColor() '''

description:

Color recognition, get the color of the current video stream frame parameter:

Mode:1star, Run a frame

return:

r,g,b: colour gamut

state:0 failed 1succeed

'''

Example code: ret = api.single_fly_getColor()#return：r,g,b: colour gamut state:0 failed 1 succeed

Color recognition, get the color of the current video stream frame

single_fly_lamplight(r, g, b, time, mode) '''

description:

Set light color and mode

parameter:

r,g,b: colour gamut

time: duration /s

mode: 1/ light on，2/light off，4/RGB cycle light，

16/ colorful light,32/ blink light,64/ breathing light

'''

Example code: api.single_fly_lamplight(255, 0, 0, 1, 1)# Set light color and mode

Set light color and mode, Does not block the main thread

plane_fly_generating(type, data ,reserve) '''

description:

lasing

parameter:

type = 0;	// laser: 0- single shot,1- keep shoting，2- Turn on the laser receiver，

3- Turn off the laser receiver 4- keep firing 5- Turn off laser

data = 10; // frequency，times/s，range 1-14 reserve = 100 // Laser gap, Data Range 1-255

'''

Example code: api.plane_fly_generating(0, 10, 100)# single shot api.plane_fly_generating(2, 10, 100) #Turn on the laser receiver

lasing

Laser receiver been hit

plane_fly_laser_receiving()

'''

description:

Laser receiver been hit

return:

True: Received laser

False: No laser was received

'''

Example code: api.plane_fly_laser_receiving()

Positioning QR code switch

Plane_cmd_switch_QR(type) '''

description:

Positioning QR code switch

parameter:

type:0-turn on   1- turn off

'''

Example code: api.Plane_cmd_switch_QR(0)

Shot

Plane_fly_take_photo() '''

description:

Before taking pictures, the video stream must be turned on

'''

Example code: api.Plane_fly_take_photo()#shot

Plane_cmd_switch_video(type) '''

description:

star recording

parameter:

type:// recording，0-star，1-end

Recording

'''

Example code: api.Plane_cmd_switch_video(0)# star recording

Video stream turn on

Plane_cmd_swith_rtp(type) '''

description:

video stream turn on

parameter:

type:0- turn on，1- turn off

'''

Example code: api.Plane_cmd_swith_rtp(0)# video stream turn on

Open the video stream window

single_fly_flip_rtp() '''

description:

Open the video stream window (the video stream must be turned on)

parameter:

'''

Example code: api.single_fly_flip_rtp()#Open the video stream window

Set the front camera tilt Angle

Plane_cmd_camera_angle(type, data) '''

description:

Set the front camera tilt Angle

parameter:

type = 0;	// direction: 0-up,1-down(absolutely), 2 and 3 algorithm control，

4- calibration，5- scratch block up，6- scratch block down（relatively）

data = 30; // Angle range: 0~90

'''

Example code: api.Plane_cmd_camera_angle(0, 30)# Set the front camera tilt Angle

Unlock the

plane_fly_arm() '''

Low speed propeller

description:

Unlock drone motor propeller

parameter:

'''

Example code: api.plane_fly_arm()#Low speed propeller

Lock the

plane_fly_arm() '''

Low speed propeller

plane_fly_disarm() '''

description:

turn off drone motor

parameter:

'''

Example code: api.plane_fly_disarm()#Lock the Low speed propeller

Get obstacle avoidance direction information

Plane_getBarrier() '''

description:

Get obstacle avoidance direction information

parameter:

return: Dictionary Obstacle status in each direction

，True: There is an obstacle，False: There is No obstacle

{

'forward': True

'back': True,

'left': True,

'right': True,

}

'''

Example code: ret = api.Plane_getBarrier()#Get obstacle avoidance direction information

get_battery() '''

description:

Get drone battery percentage

returned value:

Integer: battery percentage

'''

Example code: ret = api.get_battery()#Get drone battery percentage

Get drone battery percentage

get_coordinate() '''

description:

Get drone coordinates [x, y, z]

parameter:

returned value:

[x, y, z]

'''

Example code: ret = api.get_coordinate()#Get drone coordinates [x, y, z]

Get drone coordinates (x,y,z)

get_yaw() '''

description:

Get drone Angle（degree）

returned value:

Integer :[yaw Angle, pitch Angle, roll Angle]

'''

Example code: ret = api.get_yaw()

Get drone Angle

get_plane_speed() '''

description:

Get drone speed (X axis speed,Y axis speed,Z axis speed)

returned value:

Integer:[X,Y,Z]

'''

Example code: ret = api.get_plane_speed()

Get drone speed (X axis speed,Y axis speed,Z axis speed)

Get the drone Tof height value

get_plane_distance() '''

description:

Get the drone Tof height value

returned value:

Integer: drone Tof height value

'''

Example code: ret = api.get_plane_distance()

Get drone ID

get_plane_id() '''

description:

Get drone ID

返回值:

Integer: drone ID

'''

Example code: ret = api.get_plane_id()

External cramp，electromagnet

Plane_cmd_clamp(type,angle) '''

description:

External cramp，electromagnet

parameter:

type: 0: Clamp disable,1: Clamp enable,2: Clamp Angle,3: Electromagnet pops out,4: Electromagnet attracts

'''

Example code 1：cramp
api.Plane_cmd_clamp(1)  # Enable the cramp first‌ – angle control becomes effective afterward.
api.Plane_cmd_clamp(type=2,angle=90)  # Retrieve current cramp angle → ‌output its 0° reference         position‌.

time.sleep(5) # Wait for 5s
api.Plane_cmd_clamp(2,0) # Retrieve current cramp angle → ‌output its 90° reference position‌.
time.sleep(5) # Wait for 5s
api.Plane_cmd_clamp(0)  # Disable the cramp ‌ – angle control is deactivated afterward.

Example code 2：electromagnet
api.Plane_cmd_clamp(type=4)  # Electromagnet attracts
time.sleep(5) # Wait for 5s
api.Plane_cmd_clamp(type=3) # Electromagnet pops out